AXI4-Lite Peripheral Controller

Project motivation, team plan, module descriptions and pin-width specification

# 1. Why AXI4-Lite matters

AXI (Advanced eXtensible Interface) is part of Arm's AMBA family of on-chip communication standards. It defines how IP blocks exchange addresses, data and responses inside a System-on-Chip (SoC). Processors, DMA engines, accelerators, memory controllers and peripherals need predictable interfaces so that independently developed blocks can be integrated and reused.

AXI-family interfaces are used in SoC and FPGA designs. AXI4 supports high-throughput memory-mapped transactions, while AXI4-Lite is a simpler memory-mapped interface often used to configure and monitor peripherals. Software can write a control register, read a status register, set an interrupt mask or inspect a counter. These concepts are relevant to embedded control, networking, signal processing, accelerators, storage, automotive and industrial systems.

This project is intentionally smaller than a complete AXI4 system but still requires correct handling of five independent channels (AW, W, B, AR, R), VALID/READY handshakes, independent write-address and write-data arrival, backpressure, errors, reset and simultaneous read/write activity. Registers, byte strobes, interrupts and a CDC event counter make it more realistic than a simple register demo.

# 2. CV and skill value

A complete implementation can demonstrate RTL design, protocol understanding, self-checking verification, debugging, lint, CDC analysis, synthesis constraints, timing/area analysis and DFT concepts. The strongest CV evidence is a working repository with repeatable simulation, reviewed coverage, and real tool reports that each team member can explain. This is not a job guarantee; results and understanding matter more than the project title.

| Quality target | Requirement |
| --- | --- |
| Functional coverage | At least 98% of the approved functional coverage model; review exclusions. |
| Code coverage | At least 96% for each agreed metric supported by the simulator, such as statement, branch, condition and toggle. |
| Regression | All required tests pass; keep seeds, logs, assertions and failure summaries. |

# 3. Team responsibilities and coding rules

| Person | Main ownership | Required local verification |
| --- | --- | --- |
| Zyad | 40% RTL: AXI slave; top integration. ASIC flow: lint, CDC, DFT/scan where supported, synthesis and STA. 40% of system DV. | Test AXI handshakes, AW/W ordering, response stalls, reset and a top-level smoke test; own protocol assertions and regression integration. |
| Nourhan | Register bank, address decode, read mux and byte strobes. | Self-checking tests for reset values, every register, RO/RW rules, byte strobes and invalid addresses. |
| Mohamed Ali | IRQ controller, event counter, CDC synchronizer and reset synchronizer. | Self-checking tests for IRQ set/mask/clear, counter increments/wrap, reset and documented CDC assumptions. |
| Islam | 60% of system DV: SystemVerilog testbench, BFM, scoreboard, tests, coverage and regression. | Build reusable AXI transactions and full-system tests; integrate assertions/coverage with Zyad. |

All synthesizable RTL must be Verilog-2001 in .v files. Verification uses SystemVerilog in .sv files. Every RTL owner must supply a small self-checking module-level testbench that drives inputs, checks outputs, reports pass/fail and has a timeout. These tests catch local bugs before integration and make later UVM migration easier. UVM will still require proper agents, sequences, driver/monitor, scoreboard/reference model and coverage integration; the simple tests do not automatically convert to UVM.

Module Specification

Responsibilities, ports, widths and connection rules

# 4. Architecture overview

AXI master/testbench ⇄ axi4lite_slave_if ⇄ alpc_reg_bank

├── alpc_irq_ctrl → irq

evt_clk/event_pulse → event_counter → cdc_gray_sync → alpc_reg_bank readback

Each clock domain uses its own reset_sync instance. alpc_top only instantiates and connects modules.

# 5. Top-level pins: alpc_top.v

| Port | Dir. | Width | Description |
| --- | --- | --- | --- |
| s_axi_aclk | Input | 1 | AXI/register clock. |
| s_axi_aresetn | Input | 1 | Active-low reset for bus/register domain; async assertion, synchronized release. |
| evt_clk | Input | 1 | Independent event-counter clock. |
| evt_aresetn | Input | 1 | Active-low reset for event domain; async assertion, synchronized release. |
| event_pulse | Input | 1 | Event input synchronous to evt_clk; each sampled-high cycle increments the counter. |
| irq | Output | 1 | Level interrupt asserted when a pending enabled interrupt is present. |
| s_axi_awaddr | Input | 8 | Write byte address; legal offsets 0x00–0x3C, word aligned. |
| s_axi_awvalid | Input | 1 | Write address valid. |
| s_axi_awready | Output | 1 | Write address accepted. |
| s_axi_wdata | Input | 32 | Write data. |
| s_axi_wstrb | Input | 4 | Byte write enables; bit n controls WDATA[8n +: 8]. |
| s_axi_wvalid | Input | 1 | Write data valid. |
| s_axi_wready | Output | 1 | Write data accepted. |
| s_axi_bresp | Output | 2 | Write response: 2'b00 OKAY; 2'b10 SLVERR. |
| s_axi_bvalid | Output | 1 | Write response valid. |
| s_axi_bready | Input | 1 | Master accepts write response. |
| s_axi_araddr | Input | 8 | Read byte address; legal offsets 0x00–0x3C, word aligned. |
| s_axi_arvalid | Input | 1 | Read address valid. |
| s_axi_arready | Output | 1 | Read address accepted. |
| s_axi_rdata | Output | 32 | Read data. |
| s_axi_rresp | Output | 2 | Read response: 2'b00 OKAY; 2'b10 SLVERR. |
| s_axi_rvalid | Output | 1 | Read response valid. |
| s_axi_rready | Input | 1 | Master accepts read response. |

Handshake rule: a transfer occurs on a rising edge where VALID and READY are both high. The source must keep VALID and payload stable until acceptance. AW and W can arrive in either order. Only legal aligned offsets are implemented; unaligned/unmapped accesses and writes to read-only registers return SLVERR.

# 6. Module descriptions and module-level interfaces

## 6.1  axi4lite_slave_if.v — owner: Zyad

Implements the AXI4-Lite protocol-facing control. Captures AW and W independently, issues a write to the register bank only when both parts are available, returns B responses, captures AR, and holds RDATA/RRESP until RREADY. Read and write channels operate independently. The register-side interface below is a recommended contract; freeze names and timing before parallel coding.

| Port | Dir. | Width | Description |
| --- | --- | --- | --- |
| aclk | Input | 1 | Bus clock. |
| aresetn | Input | 1 | Active-low bus reset. |
| s_axi_awaddr | Input | 8 | AXI write address. |
| s_axi_awvalid | Input | 1 | Write address valid. |
| s_axi_awready | Output | 1 | Address handshake. |
| s_axi_wdata | Input | 32 | AXI write data. |
| s_axi_wstrb | Input | 4 | Write byte enables. |
| s_axi_wvalid | Input | 1 | Write data valid. |
| s_axi_wready | Output | 1 | Write data handshake. |
| s_axi_bresp | Output | 2 | Write response code. |
| s_axi_bvalid | Output | 1 | Write response valid. |
| s_axi_bready | Input | 1 | Write response ready. |
| s_axi_araddr | Input | 8 | AXI read address. |
| s_axi_arvalid | Input | 1 | Read address valid. |
| s_axi_arready | Output | 1 | Read address handshake. |
| s_axi_rdata | Output | 32 | Read response data. |
| s_axi_rresp | Output | 2 | Read response code. |
| s_axi_rvalid | Output | 1 | Read response valid. |
| s_axi_rready | Input | 1 | Read response ready. |
| reg_wr_en | Output | 1 | One-cycle write commit pulse after AW and W are both accepted. |
| reg_wr_addr | Output | 8 | Captured write address. |
| reg_wr_data | Output | 32 | Captured write data. |
| reg_wr_strb | Output | 4 | Captured byte enables. |
| reg_wr_err | Input | 1 | Register bank indicates invalid address or prohibited write. |
| reg_rd_en | Output | 1 | Read request/lookup indication; exact pulse timing defined in interface note. |
| reg_rd_addr | Output | 8 | Captured read address. |
| reg_rd_data | Input | 32 | Register bank read data. |
| reg_rd_err | Input | 1 | Register bank indicates invalid read address. |

## 6.2  alpc_reg_bank.v — owner: Nourhan

Contains the 16-word register map, address decode, write permission checks, byte strobes, read mux and error generation. Suggested contract: synchronous writes on aclk; combinational read data from reg_rd_addr, captured by the AXI interface when the read is accepted. A registered-read alternative is acceptable if both modules and tests agree on latency.

| Port | Dir. | Width | Description |
| --- | --- | --- | --- |
| aclk | Input | 1 | Register clock. |
| aresetn | Input | 1 | Active-low register reset. |
| wr_en | Input | 1 | Write commit request. |
| wr_addr | Input | 8 | Write byte address. |
| wr_data | Input | 32 | Write payload. |
| wr_strb | Input | 4 | Byte write enables. |
| wr_err | Output | 1 | High for unaligned/unmapped address or illegal RO write. |
| rd_addr | Input | 8 | Read byte address. |
| rd_data | Output | 32 | Selected register/read-only value. |
| rd_err | Output | 1 | High for unaligned/unmapped read address. |
| event_count | Input | 32 | Synchronized event counter from CDC block. |
| irq_status | Input | 32 | Pending interrupt status from IRQ block, if the chosen architecture exposes it. |
| irq_mask | Output | 32 | Interrupt enable/mask register value, if controlled by register bank. |
| irq_status_w1c | Output | 32 | Write-one-to-clear mask/pulse, if W1C decode is separated into IRQ block. |

Register map: 0x00 ID (RO, 0x414C5043); 0x04 VERSION (RO, 0x00010000); 0x08 CONTROL (RW); 0x0C STATUS (W1C/RO readback); 0x10 IRQ_MASK (RW); 0x14 EVENT_COUNT (RO); 0x18–0x3C SCRATCH0–SCRATCH9 (RW).

## 6.3  alpc_irq_ctrl.v — owner: Mohamed Ali

Maintains sticky interrupt status, applies the mask and drives irq. Agree on event-set versus W1C-clear priority; recommended priority is set wins if a new event and clear target the same bit in the same cycle.

| Port | Dir. | Width | Description |
| --- | --- | --- | --- |
| aclk | Input | 1 | Bus clock. |
| aresetn | Input | 1 | Active-low reset. |
| event_set | Input | 1 | Synchronized event pulse or event-pending indication. |
| status_w1c | Input | 1 | Write-one-to-clear pulse for the event status bit. |
| irq_mask | Input | 1 | 1 enables this interrupt; 0 masks it. |
| status | Output | 1 | Sticky pending event status. |
| irq | Output | 1 | status AND irq_mask. |

## 6.4  event_counter.v — owner: Mohamed Ali

Counts sampled-high event_pulse cycles in evt_clk domain. Counter wraps modulo 2^32. Converts binary count to Gray code for safe multi-bit CDC transfer.

| Port | Dir. | Width | Description |
| --- | --- | --- | --- |
| evt_clk | Input | 1 | Event clock. |
| evt_aresetn | Input | 1 | Active-low event-domain reset. |
| event_pulse | Input | 1 | Synchronous event input. |
| count_bin | Output | 32 | Binary counter, for local debug/use only if needed. |
| count_gray | Output | 32 | Gray-coded counter sent to synchronizer. |

## 6.5  cdc_gray_sync.v — owner: Mohamed Ali

Synchronizes Gray-coded counter bits into the bus clock domain using two destination-clock stages, then converts the synchronized Gray value back to binary. Apply tool-recognized synchronizer attributes and CDC constraints.

| Port | Dir. | Width | Description |
| --- | --- | --- | --- |
| dst_clk | Input | 1 | Destination bus clock. |
| dst_aresetn | Input | 1 | Active-low reset synchronized for destination domain. |
| gray_async | Input | 32 | Gray count from event domain. |
| count_bin_sync | Output | 32 | Synchronized binary count read by register bank. |

## 6.6  reset_sync.v — owner: Mohamed Ali

Provides asynchronous reset assertion and synchronous deassertion for one clock domain. Instantiate separately for the bus clock and event clock, unless the project methodology specifies a different reset architecture.

| Port | Dir. | Width | Description |
| --- | --- | --- | --- |
| clk | Input | 1 | Clock for this reset domain. |
| arstn | Input | 1 | External active-low asynchronous reset. |
| srstn | Output | 1 | Active-low reset with synchronous release. |

## 6.7  tb_alpc_top.sv and unit testbenches — owner: verification / each RTL owner

Verification-only modules are SystemVerilog and are not synthesized. Each RTL owner writes a small module testbench for their block. Islam owns the shared AXI BFM, scoreboard/reference model, system tests and coverage model; Zyad owns protocol assertions and contributes 40% of system verification and regression integration.

| Verification component | Key signals / data widths | Purpose |
| --- | --- | --- |
| AXI BFM | AXI pins exactly match top-level widths | Drives valid transactions, controls READY stalls, checks bounded completion. |
| Scoreboard | Transaction address 8b, data 32b, strobes 4b, response 2b | Predicts register behavior and compares observed results. |
| Assertions | VALID/READY, payload stability, response ordering | Detects protocol violations and deadlocks. |
| Functional coverage | Register, operation, response, strobe, stall and concurrency bins | Measures scenario completion against ≥98% target. |

# 7. Register map details

| Offset | Register | Access | Reset / behavior |
| --- | --- | --- | --- |
| 0x00 | ID | RO | 0x414C5043 |
| 0x04 | VERSION | RO | 0x00010000 |
| 0x08 | CONTROL | RW | 0; bit 0 enables event/IRQ feature if enabled by final design |
| 0x0C | STATUS | W1C/read | Bit 0 event pending; writing 1 to bit 0 clears it |
| 0x10 | IRQ_MASK | RW | 0; bit 0 = 1 enables interrupt |
| 0x14 | EVENT_COUNT | RO | Synchronized 32-bit event counter |
| 0x18–0x3C | SCRATCH0–9 | RW | Reset 0; 32-bit storage each |

All addresses are byte addresses and must be 4-byte aligned. WSTRB[n] updates byte WDATA[8*n +: 8]. RO writes, unaligned addresses and unmapped addresses return SLVERR. Finalize the exact location of W1C logic and the IRQ status/mask interface before implementation so the register bank and IRQ block do not both drive the same state.

# 8. ASIC flow and project exit criteria

- RTL compiles and the top module contains only instances and connections.

- Each module owner provides a passing self-checking unit testbench before integration.

- SystemVerilog regression passes; no unexplained X/Z values; assertions are clean.

- Functional coverage ≥98%; code coverage ≥96% for each agreed metric, with exclusions reviewed.

- Lint findings fixed or documented; CDC crossings and waivers reviewed.

- DFT rule checks and scan insertion run with a suitable standard-cell library and compatible tools.

- Synthesis and STA reports include constraints, area, worst slack and unresolved warnings.

- Repository includes specification, interface contract, RTL, tests, scripts and final reports.